SPREADSHEET TOPICS ON COMPUTERISED

DESIGN OFFICE PRACTICE

Part 1: Analysis And Design of Solid Slabs to BS8110

Analysis Procedure

In the design of restrained slabs, the slab is divided in strips in both the X and Y directions.  Although the calculated moments apply only to the middle strips, it is recommended that the steel is placed uniformly throughout the slab.

The design ultimate moments given by equations 14 and 15 of the Code (BS8110) are:

msx	=	bsx nµ §x2	.	.	.	(1)

msy	=	bsy nµ §x2	.	.	.	(2)

Where

µ §x	=	length of shorter side of slab

µ §y	=	length of longer side of slab

msx 	=	maximum ultimate moment at midspan or over supports on

strips of unit width and spanning µ §x

msy	=	maximum ultimate moment at midspan or over supports on

strips of unit and spanning µ §y

bsx and bsy  	=	Moment coefficients

The values of the moment coefficient in the above equations can be determined as follows:

a)	Coefficient, by, for positive midspan moment for strips spanning µ §y

is given by equation 16 of the Code as follows:

by  =	(24 + 2Nd + 1.5 Nd2) / 1000	.	.	(3)

Where

Nd 	=	the number of discontinuous edges (0 < Nd <4)

b)	Coefficient for negative support moments for strips spanning µ §y (b1 and b2)

For a discontinuous edge the coefficient is zero.

For a continuous edge the coefficient is 4/3 of the positive midspan coefficient by.  That is

b1  = 0 or 4/3 by

b2  = 0 or 4/3 by

Where b1 and b2 are the coefficients for the negative (hogging) moments over the two shorter side lengths.

c)	Coefficient bx for positive midspan  moment for strips spanning µ §x.

bx can be determined from the following expressions given by equation 16 of the code, viz

µ §.	.	.	(4)

and equation 17 of the code, viz

µ §	.	.	.	.	(5)

Where b3 and b4 are the coefficients for the negative (hogging) moments over the two longer sides and, as for b1 and b2, take the value of zero for a discontinuous edge or 4/3 rds of the positive midspan moment bx for a continuous edge.

The coefficients b1 to b4 can be expressed as

b1 = K1by;   b2 = K2by;   b2 = K3bx   and   b4 = K4bx

Where K1 to K4 are constants which can take the value of zero or 4/3 depending on the continuity of the edge.

Hence from equation (4)

µ §	.	(6)

and from equation (5)

µ §

or   µ §

and squaring both sides

µ §

or	µ §	.	.	.	(7)

Therefore, to calculate the coefficient, bx, the following procedure may be followed:

(i)	Assign value of zero or 4/3 to constants K1 to K4 depending on the edge conditions of the slab (discontinuous or continuous respectively).

(ii)	Use equation (3) to calculate the coefficient by

(iii)	Use equation (6) to calculate g

(iv)	Use equation (7) to calculate the coefficient bx

d)	Coefficient for negative support moments for strips spanning µ §x ( b3 and  b4)

According to the edge conditions  b3 = 0 or 4/3  bx.

Likewise  b4 =  0 or 4/3 bx.

Once the moment coefficients are calculated, the maximum ultimate moments are calculated using equation (1) and (2) above.

Design Procedure

The flexural design of reinforced concrete slabs is similar to the design of reinforced concrete beams.  Once the design moments are calculated, the design of the areas of longitudinal tension reinforcement could be accomplished as follows:

a)	Calculate  b = M/bd2

Where  M   = 	design moment

b    =	breadth of slab, usually equals to 1000mm

d    =	effective depth of section.

b)	Determine the lever arm    µ §    0.95

Where fcu  =	characteristic concrete strength.

c)	Calculate As required   =   µ §

Where fy  =	characteristic steel strength.

d)	Check deflection by

(i)	Selecting the basic span / effective depth ratio

(ii)	Multiply the basic ratio by a factor which allows for the area of tension reinforcement and which is given by

modification factor  =  µ §

Where fs  =	µ §

M	   =	design moment at midspan

fy	   =	characteristic steel strength

As.req	   =	area of tension steel required at midspan

As. prov  =	are of tension steel provided at midspan

bb	=	ratio of the resistance moment at the section after re-

distribution to that before re-distribution.

e)	Estimate the reinforcement weight in Kg per square metre as given the Handbook (  ).

Estimated weight  =	0.011 (Asy + Asx)

Where Asy  =	Area of steel provided in the longitudinal direction

and      Asx  =	Area of steel provided in the transverse direction.

Having defined all the formulae required to analysed, design, check deflections and estimate reinforcement weights for solid slabs let us now create a spreadsheet program to perform the procedures:

Spreadsheet program

Step     1.	Start with a new worksheet, expanded to full size.

Type the following entries in the Specified Cells.

| 2 | A1 : Slab I.D. |  |  | Any text for slab I.D ( eg Slab AB/12) |  |
| --- | --- | --- | --- | --- | --- |
| 3 | A2 : µ §y - m |  |  | B2 : 6 |  |
| 4 | A3 : µ §x - m |  |  | B3 : 5 |  |
| 5 | A4 : Type of slab (1,2,3 or 4) |  |  | B4 : 4 |  |
| 6 | A6 : Thickness - mm |  |  | B6 : 150 |  |
| 7 | A7 : Finishes - KN/m2 |  |  | B7 : 1.5 |  |
| 8 | A8 : Live load - KN/m2 |  |  | B8 : 2.5 |  |
| 9 | A9 : Moveable partitions - KN/m2 |  |  | B9 : 1.5 |  |
| 10 | A11 : Assumed bar dia. - mm |  |  | B11: 12 |  |
| 11 | A12 : Cover - mm |  |  | B12: 20 |  |
| 12 | A14 : fcu - N/mm2 |  |  | B13: 20 |  |
| 13 | A15 : fy - N/mm2 |  |  | B14: 410 |  |
| 14 | A17 : Total serve load - KN/m2 |  |  | B17 : = 24*B6/1000 + B7 + B8 + B9 |  |
| 15 | A18 : Design load - KN/m2 |  |  | B18 : = 1.4* (24*B6/1000 +B7) + 1.6 * (B8 + B9) |  |
| 16 | A20 : Effective depth - mm |  |  | B20 : = B6 - B12 - B11/2 |  |
| 17 | A22 : Nd |  |  | B22 : = Choose (B4, 0,1,1,2) |  |
| 18 | A23 : by |  |  | B23 : = (24+2*B22+1.5*B22^2) /1000 |  |
| 19 | A24 : b1 |  |  | B24 : = If (B4 > 2, 0, 4/3*b23) |  |
| 20 | A25 : b2 |  |  | B25 : = 4/3 * B23 |  |
| 21 | A26 : g |  |  | B26 : = 2/9 * (3-Sqrt(18) * B3/B2 * (Sqrt(B23+B24) + Sqrt(B23 + B25))) |  |
| 22 | A27 : K3 |  |  | B27 : = 4/3 |  |
| 23 | A28 : K4 |  |  | B28 : = If(MOD(B4,2)= 0,0,4/3) |  |
| 24 | A29 : bx |  |  | B29 : = B26/(Sqrt (1+B27) + Sqrt (1+B28)) ^2 |  |
| 25 | A30 : b3 |  |  | B30 : = B27 * B29 |  |
| 26 | A31 : b4 |  |  | B31 : = B28 * B29 |  |
| 27 | C21 : Coefficient |  |  | D21 : = B23 |  |
| 28 | C22 : Moment |  |  | D22 : = D21 * $B$18*$B$3^2 |  |
| 29 | C23 : M/bd2 |  |  | D23 : = D22 * 1000/$B$20^2 |  |
| 30 | C24 : Z/d |  |  | D24 : = Min (0.95,0.5+Sqrt(0.25-D23/0.8/$B$14)) |  |
| 31 | E21 : = B24 F21 : = B25 G21 : = B29 H21 : = B30 I21 : = B31 |  |  |  |  |
| 32 | Select Cells D22:D24 and copy into E21: I24 |  |  |  |  |
| 33 |  |  | D2: Long |  |  |
| 34 |  |  | D3: Span |  |  |
| 35 | C5 : As req |  | D5: = D22*1e6/(0.87*$B$15*D24*$B$20) |  |  |
| 36 | C7 : bar dia. |  | D7 : 12 |  |  |
| 37 | C8: spacing |  | D8 : 300 |  |  |
| 38 | C10 : As prov. |  | D10 : = P1()* D7^2 /D8 * 1000 |  |  |
| 39 | C11 : 100 As/bd |  | D11 : = D10/10/$B$20 |  |  |
| 40 | E2 : Long F2 : Long . E2 : Short H2 : Short I2 : Short |  |  |  |  |
| 41 | E3 : Support1 F3 : Support2 E3 : Span H3 : Support1 I3: Support2 |  |  |  |  |
| 42 | Select cells D5 : D11 and Copy into E5 : I11 |  |  |  |  |
| 43 | C13 : basic l/b |  | D13 : 26 |  |  |
| 44 | C26 : fs |  | D26 : = 5/8 *D5/D10 * $B$15 |  |  |
| 45 | C27 : mf |  | D27 : = Min (2, 0.55+(477-D26) / (120*(0.9 + D23))) |  |  |
| 46 | C28 : allowed l/d |  | D28 : = D27 * D13 |  |  |
| 47 | C29 : actual l/d |  | D29 : = $B$2/$B$20 * 1000 |  |  |
| 48 | C14 : Deflection | D14 : = If (D28 > D29,"Okay","Not Okay") |  |  |  |
| 49 | Select D13 : D14 and Copy into G13 : G14 |  |  |  |  |
| 50 | Select D26 : D29 and Copy into G26 : G 29 |  |  |  |  |
| 51 | Edit G29 to read $B$3/$b$20*1000 |  |  |  |  |
| 52 | C16 : Wt. Kg/m2 D16 : = 0.011 * (D10 + G10) |  |  |  |  |

NOTE:

Type of slab restrained coding.

µ §

TYPE		Description

1		All edges continuous

2		One long-edge discontinuous

3		One short-edge discontinuous

4		One long and one short edges discontinuous.

Then we can use a look up functions

Chose (Type of slab, 0,1,1,2)

to assign the number of discontinuous edges, Nd.